[CEN 323 ] Computer Organization & Assembly Language

Lab 12    Journal

Name :Bilal Aamir

Enrollment Number:01-135232-016

# Lab Deliverables (submission guideline)

- Create a folder named after your enrollment number and add the following files:

  - Create a folder named YourRollNumber_Lab12 containing:

- .asm files of the 2 practice tasks.

- Add full screen screenshots of the code and output in this word file.

  - Zip the Folder and upload to LMS.

# Lab Task

## Task 1:

Code (TEXT):

INCLUDE EMU8086.INC

ORG 100H

.STACK 100H

.DATA

ARR1    DW 1,2,3,4,5

ARR2    DW 6,7,8,9,1

COUNT   DW 5

RESULT1 DW 5 DUP(?)

RESULT2 DW 5 DUP(?)

.CODE

COPYARRAY PROC

PUSH BP

MOV  BP, SP

MOV  CX, [BP+4]

MOV  DI, [BP+6]

MOV  SI, [BP+8]

STORELOOP:

MOV  AX, [SI]

MOV  [DI], AX

ADD  SI, 2

ADD  DI, 2

LOOP STORELOOP

POP  BP

RET  6

COPYARRAY ENDP

MAIN:

PUSH OFFSET ARR1

PUSH OFFSET RESULT1

PUSH COUNT

CALL COPYARRAY

PUSH OFFSET ARR2

PUSH OFFSET RESULT2

PUSH COUNT

CALL COPYARRAY

MOV  AX, 4C00H

INT  21H

Screenshot (CODE):

Screenshot (OUTPUT):

## Task 2:

Code (TEXT):

INCLUDE EMU8086.INC

.STACK 100H

.DATA

ARR1    DW 1,2,3,4,5

ARR2    DW 6,7,8,9,1

COUNT   DW 5

MSG1    DB 'ARR1 replacements: $'

MSG2    DB 'ARR2 replacements: $'

.CODE

REPLACEVAL PROC

PUSH BP

MOV  BP, SP

SUB  SP, 2

MOV  [BP-2], 0

MOV  CX, [BP+6]

MOV  SI, [BP+8]

MOV  DX, [BP+4]

SCANLOOP:

MOV  AX, [SI]

CMP  AX, DX

JNE  SKIP

MOV   [SI], 0

INC   [BP-2]

SKIP:

ADD  SI, 2

LOOP SCANLOOP

MOV  AX, [BP-2]

MOV  SP, BP

POP  BP

RET  6

REPLACEVAL ENDP

MAIN:

MOV AX,@DATA

MOV DS,AX

PUSH ARR1

PUSH COUNT

PUSH 3              ;

CALL REPLACEVAL

MOV AH, 09H

MOV DX, OFFSET MSG1

INT 21H

ADD AL, '0'

MOV DL, AL

MOV AH, 02H

INT 21H

PRINTN ""

PUSH OFFSET ARR2

PUSH COUNT

PUSH 9

CALL REPLACEVAL

MOV AH, 09H

MOV DX, OFFSET MSG2

INT 21H

ADD AL, '0'

MOV DL, AL

MOV AH, 02H

INT 21H

PRINTN ""

MOV SI, OFFSET ARR1

MOV CX, 5

PRINT_R1:

MOV AX, [SI]

ADD AL, '0'

MOV DL, AL

MOV AH, 02H

INT 21H

MOV DL, ' '

MOV AH, 02H

INT 21H

ADD SI, 2

LOOP PRINT_R1

PRINTN ""

MOV SI, OFFSET ARR2

MOV CX, 5

PRINT_R2:

MOV AX, [SI]

ADD AL, '0'

MOV DL, AL

MOV AH, 02H

INT 21H

MOV DL, ' '

MOV AH, 02H

INT 21H

ADD SI, 2

LOOP PRINT_R2

MOV AX, 4CH

INT 21H

END MAIN

Screenshot (CODE)

Screenshot (OUTPUT):